- Restricted -

Copying of this document and giving it to others and the use or communication of the contents thereof are forbidden without express authority. Offenders are liable to the payment of damages. All rights are reserved in the event of the grant of a patent or the registration of a utility model or design.

AR_7SR5_Platform_SW.doc

| Product / Function: | 7SR5 SW Platform |
| --- | --- |

Author

| Name | Department | Version | Date |
| --- | --- | --- | --- |
| Neil Dosdale | EM DG SPDL R&D | V1.00 | 2017-11-20 |
| Neil Dosdale | EM DG SPDL R&D | V1.03 | 2018-03-01 |
| Neil Dosdale | SI DG SA&P PR GB-R&D 2 | V1.04 | 2019-07-04 |
| Steven Idle | SI EA R&D MP GB 1 | V1.05 | 2022-11-14 |

Updates

| Chapters / Pages changed | Version | Object and reason of change / Reference to change requirements |
| --- | --- | --- |
| - | V1.00 | Renamed and setup to the main AR / design reference manual for the firmware on the RN platform. |
|  | V1.03 | Added the A9 and KON file programming information. Updated information for debugging and connecting using the SEGGER J-Link. |
|  | V1.04 | + Converted from original project specific file into a generic 7SR5 software platform document. + Added the firmware signing process. + Added the M4 memory map. + Updated J-Link tool version to 6.34h + Added Deliverables, File Formats and Loading Mechanisms. + Added MAC addresses and Firmware Signing. |
| Table 6 1 | V1.05 | + Updated Table 6 1 Slave Card Article Numbers |

Contents

1	Preface	6

1.1	General notes	6

1.2	Business case	6

1.3	Terms and definitions	6

2	Requirements and architectural drivers	7

2.1	Functional requirements	7

2.2	Non functional requirements (NFR)	7

2.3	Further architectural drivers	7

3	Architecture description	8

3.1	A9 Architecture	9

3.1.1	Boot-Rom Component	10

3.1.2	50BSystem-Base Component	10

3.1.3	51BApplication Component	11

3.1.4	Boot-Rom Initial	11

3.1.5	Application Initial	12

3.1.6	A9  - Firmware Update process	13

3.2	M4 Firmware Architecture	13

3.2.1	Binary Application	14

3.2.2	RDL script files	16

3.2.3	SPI Interface	17

3.2.4	A9 core Interface	18

3.2.5	M4 Flash - Memory Map	18

3.2.6	Third party components	19

3.3	7SR5 Device Architecture	19

3.4	Reydisp Manager Template Generation	19

3.4.1	Template Input Format	19

3.4.2	Template Output Format	19

3.4.3	RDS/RDO Processing	19

3.4.4	Dynamic SCL	24

4	General Architectural Topics	25

4.1	Release Classification & Version Planning	25

4.2	File Formats – CMS/VOL/KON/PCK	25

4.2.1	File Formats – CMS	25

4.2.2	File Formats – VOL	25

4.2.3	File Formats – KON	25

4.2.4	File Formats – PCK	26

4.3	Deliverable Packages	26

4.3.1	Deliverable Packages – Protection Firmware	26

4.3.2	Deliverable Packages – Communications Firmware	26

4.3.3	Deliverable Packages – User Configurations	26

4.3.4	Deliverable Packages – Production	26

4.4	Device Upload – Firmware and configurations	28

4.4.1	DTLS loader – Recovery Mechanism (FMU)	28

4.4.2	HTTPS loader – Device Healthy upload mechanism	28

4.5	Firmware Signing & Verification	28

4.5.1	Firmware Signing – Overview	29

4.5.2	Firmware Signing – File Formats	29

4.5.3	Firmware Signing – PC Setup	29

4.5.4	Firmware Signing – Signing Process	29

4.5.5	Firmware Signing – Disable Firmware Signing (For Development Only)	30

4.6	MAC Addresses	30

4.6.1	MAC Addresses – Creating a MAC in development	30

4.6.2	MAC Addresses – HBB R&D Assigning a MAC address	30

4.7	Watchdog Mechanism	31

4.8	Exception handing and error processing	31

4.9	Endian Format	32

5	A9 Firmware Architectural Topics	33

5.1	A9FW – CMP	33

5.2	A9FW – Board Programming	33

5.2.1	Prerequisite Tools	33

5.2.2	KON file loader	33

5.2.3	Boot Fuses	33

5.2.4	Boot ROM	34

5.2.5	FPGA	34

5.2.6	Network Settings	34

5.2.7	VxWorks	34

5.2.8	M4 Binary	34

6	M4 Firmware Architectural Topics	35

6.1	M4FW – CMP	35

6.1.1	Software Project Shell (SPS)	35

6.1.2	IAR Compiler	35

6.1.3	J-Link Debugging Tools	36

6.2	M4FW – Slave Card Programming	36

6.2.1	Basic Card Programming	36

6.2.2	Analogue Card Programming	36

6.2.3	Slave Card Article Numbers	36

6.3	M4FW – Sampling	37

6.3.1	Analogue Input RDL naming convention	37

6.4	M4FW – Timers	37

6.5	M4FW – Hardware timers	37

6.6	M4FW – Real Time Clock	37

7	7SR5 Devices Architectural Topics	39

8	Reydisp Manager Architectural Topics	41

8.1	Reydisp Manager Template Dataflow	41

8.2	Dynamic Setting View/Edit	41

8.3	Complex Settings	44

8.3.1	I/O Matrix Bitfield settings	44

8.3.2	Or Masks (Pickup Config) Settings	45

8.4	Reydisp Manager 7SR5 Template Workflow	46

9	References	47

Figures

Figure 1 – Basic Architecture	9

Figure 2 A9 Architecture	10

Figure 3 Boot-Rom initial	12

Figure 4 Application Initialize	13

Figure 5 – M4 Architecture	14

Figure 6 – Logical structure of M4 binary	15

Figure 7 – Logical structure and interconnections of RDL scripts	16

Figure 8 – Fascia Register Memory	17

Figure 9 – M4 Flash - Memory Map	18

Figure 10 – Manual configuration of the IAR network license.	35

Figure 11 - 7SR5/RDM Settings Sequence	44

Tables

Table 1- Terms, acronyms and definitions	6

Table 3-1 Module list of System-Base	11

Table 3-2 Module list of Application	11

Table 3-3: Third party components	19

Table 4-1 Manufacturing hex file deliverables	27

Table 4-2 Firmware Signing locations	29

Table 6-1 Slave Card Article Numbers	37

Table 6-2 Analogue Input RDL naming convention	37

# Preface

The current 7SR1 platform comprises two architectures:

- E4 with TC1130 or NXP LPC4437 CPU

- E6 with TC1130 CPU, with optional NXP LPC4337 slave processor for protection comms

The TC1130 CPU is obsolete and the NXP is unsuitable for native IEC 61850, due to limited internal Flash memory, and no cache for external memory.

The 7SR2 platform uses a TC1796 processor (formerly a TC1775), and there are no suitable upgrades which have Ethernet capability.  Despite a DTC exercise, the 7SR2 platform is still more expensive than the 7SR1 for similar functionality.

It is therefore proposed that a re-design of the platform is required to address these issues.  At the same time a DTC exercise will be carried out to reduce the HK cost of IEC61850 versions by at least 15%.

## General notes

## Business case

## Terms and definitions

| Term, acronym | Description |
| --- | --- |
| 7SR5 | Reyrolle Native IEC 61850 |
| RC | Reyrolle Compact |
| RDL | Relay Description Language |
| RDO | Relay Description Object |
| RDS | Relay Description Language Script |
| HWT | Hardware test code |
| HAL | Hardware Abstraction Layer |
| HMI | Human Machine Interface |
| SCL | Substation Configuration Language (IEC 61850) |
| ICD | IED Capability Description (IEC 61850) |
| XSL | XML Stylesheet Language |
| XSLT | XSL Transformations v1.0 |
| RSF | Relay/Reyrolle Setting File/Format |

Table 1- Terms, acronyms and definitions

# Requirements and architectural drivers

## Functional requirements

List the relevant requirements for the architecture from the requirements specification.
Update the list of requirements if necessary when new or changed requirements turn up during the lifetime of the product.

## Non functional requirements (NFR)

Provide a ranked list the most meaningful NFR’s as compliance, efficiency, extensibility, interoperability, maintainability, performance, portability, reliability, resource constraints, safety, security, scalability, testability.

Please refer to (http://en.wikipedia.org/wiki/Non-functional_requirement)

## Further architectural drivers

List further issues which influencing the architecture such as standards, patents, re-used components, skills, time line, budget, multi-side effects.

# Architecture description

The current design architecture used in the RC family of protection devices with IEC61850 requires a two processor system.  The main processor (Tricore TC1130) handles all functionality except the IEC61850 protocol stack.  This includes the protection functionality, the HMI, non IEC61850 comms, and all the low level drivers for accessing the binary inputs, outputs, keys and LEDs.  To provide IEC61850 comms an EN100(+) module is used and the interface between the TC1130 and the EN100 is through dual-port RAM provided by the EN100(+).  This is an 8K interface and is described in another document – see ?????

To minimize risk and delay the new software architecture will follow this same principal.  However, instead of using two microcontrollers and communicating through a dual-port RAM interface the system will run on one microcontroller (the NXP iMX6 SX) and utilize both the M4 core and the A9 core of the device.  The M4 core will provide the same functionality as the TC1130 processor in the RC platform and the A9 core will provide the same functionality as the EN100(+).

To ensure minimal changes for porting the RC platform the dual-port RAM off the EN100 will be located in an area of internal RAM that the two cores can communicate through in exactly the same way as the EN100.  In effect, this makes the A9 core look and behave like an EN100 in respect to its memory interface.

The TC1130 port of the RC will not be used in this case.  A cost reduced version of the RC platform uses the NXP LPC4337 microcontroller that uses an M4 core and the port for the iMX6 SX will use this port – hopefully the libraries can just be used directly although some investigation will be required.  If this is not possible then the libraries will need recompiling for the iMX6 SX.

The IEC61850 stack used by the EN100(+) has already been ported and is used on an A9 core.  Similarly with the M4 port it is hoped that minimal work will be required for the A9 port although the endianess of the port may require some careful consideration.

Access to other cards is performed by the M4 core and uses SPI interfaces to implement a proprietary protocol allowing cards to be controlled and information shared between the main controller and all the slave cards.

The basic architecture for the 7SRn is show in Figure 1.

Figure 1 – Basic Architecture

## A9 Architecture

Upon start up the iMX6 SX boots from the A9 core.  It must initialize itself and any peripherals it is going to use, including all memories.  Once initialized the A9 core will use the USB port to determine if new code is being sent to the device and if so will then program the code received.  Finally the A9 core will determine if the main application firmware needs to be started or if HWT must be executed and then start the relevant program on the M4 core.

Once the M4 core has started the A9 core will then proceed to start either its own application code or enter HWT mode ready for the M4.

Figure 2 shows the A9 components. The basic architecture is same with EN100. Most modification causes by RTOS which changes from PSOS to Vxworks.

Figure 2 A9 Architecture

### Boot-Rom Component

The Boot-Rom based on Vxworks BSP. Main features include:

- Firmware update

- Upload image via network while some key has been pushed

- Jump to HWT mode while connecting with monitor tool(this function should be further discussion)

### 50BSystem-Base Component

The component defines as below table:

| Component Name | Description | Compare with EN100 and Smart-Device |
| --- | --- | --- |
| LIVE | Monitor other task and kick off watch-dog, task priority is highest | Same with EN100 |
| HINT | Back ground supervision, task priority is lowest | Same with EN100 |
| ANLA | Start up | Adjusting based on EN100 |
| DPR | Information exchange with M4 core, including three tasks: DPRI, DPRS, DPRR | Adjusting based on EN100 |
| HWT | Hardware-Test | Same with EN100 |
| PRX | Include three tasks: - PRX1 HTTP-Proxi for Stack in device - PRX2 RPC-Proxi for Stack in device - PRX3 DIGSI-Proxi for Stack in device PRX2/3 will be canceled due to they are not used for 7SR | Adjusting based on EN100 |
| SFS | Siemens Files System | Same with EN100 |
| EES | FPGA Diagnosis & Control | Adjusting based on EN100 |

Table 3-1 Module list of System-Base

### 51BApplication Component

Application component includes a variety of protocol, such as SNMP, RSTP, IEC-61850 and so on.

| Component Name | Description | Compare with EN100 and Smart-Device |
| --- | --- | --- |
| SNTP | Simple Network Time Protocol | Adjusting based on Smart-Device |
| RSTP | Rapid Spanning Tree Protocol | Adjusting based on EN100 |
| SNMP | Simple Network Management Protocol | Adjusting based on Smart-Device |
| IEC61850 | IEC61850 protocol, including MMS and GOOSE | Adjusting based on Smart-Device |

Table 3-2 Module list of Application

### Boot-Rom Initial

The device will start up after while power on. Basic work flow of Boot-Rom defines as below.

Figure 3 Boot-Rom initial

### Application Initial

The basic work flow of application defines as below.

Figure 4 Application Initialize

### A9  - Firmware Update process

The A9 firmware update process is described in Section 5.2, page 33.

## M4 Firmware Architecture

The same design, construction, and mode of operation currently used in the RC products will be adhered to. This is shown in Figure 5.  Both the firmware and the HWT are loaded into the product during the production phase and execution of either can be selected.  It must be noted however, that only one can run at a time; either application firmware which is the default mode of operation, or HWT which is the test mode of operation.  If no application firmware is present, only HWT, then HWT will always be started.  Neither the firmware nor HWT depends on the other existing.  In order to jump from one to the other a restart of the device is required.

To start the M4 core the A9 core must first boot up and decide which program to run – HWT or the application firmware.

It will be the responsibility of the M4 core to initialize all the peripherals it requires – i.e GPIO ports, SPI, UARTS etc.  Also after starting it must gain control of the USB from the A9 core so that the front port can be used during normal operation.  Once started the M4 core will run independently from the A9 core.

Figure 5 – M4 Architecture

The RC reusable architecture has two main components.

- Binary application

- RDL script files

Following subsections describe these components

### Binary Application

The binary is the application or executable code that provides all the generic features for the device; to turn the device into a product such as the 7SR10, 7SR11, 7SR12 and so on. A set of device configuration files need to be installed into the device.

The binary has been divided into several layers, each containing logical blocks that make it easier to understand and to simplify integration and testing.

Figure 6 – Logical structure of M4 binary

- The HAL provides a basic interface onto the actual hardware.

- The Operating System (OS) is responsible for the management and coordination of activities and the sharing of the resources within the device. This includes file manager, memory manager, task manager and screens module.

- The Service Layer provides the generic frameworks that are common across all products on the RC platform. This includes sampling, HMI, communications, settings, instruments, faults etc.

- The Device Layer provides the frameworks that allow customization for the final products. These include the RDL engine, QuickLogic, ReyLogic and ReyMimic.

### RDL script files

RDL scripts are part of device layer. Most of the protection and control functions required in relays are created as block elements. These elements are written in C code which is platform/controller independent. Then these elements are provided as graphical blocks to a tool named as “Reylogic toolbox”. Through this software logical block diagrams are created with possible parameterization that is set through device settings. These scripts are added on top of the basic Binary application to turn device into a product such as 7SR10, 7SR11 and so on.

Figure 7 – Logical structure and interconnections of RDL scripts

### SPI Interface

Interaction between the iMX6 SX and the hardware is done via an SPI interface using a proprietary protocol of which a detailed explanation can be found in the Architecture document for the hardware.  The protocol uses a register based access method to enable control of the attached cards.  Each card must publish its own register set.  Figure 8 shows an example definition of a register memory layout.

Flash programming of the cards can be done in-situ and will be controlled by the M4 core.

Figure 8 – Fascia Register Memory

### A9 core Interface

As mentioned previously the A9 core will be used to implement an equivalent of the EN100(+) module so that  native IEC61850 can be provided.  To minimize changes within the M4 port the interface to the A9 core will be through an equivalent version of the EN100(+) dual-port RAM implementation.  This dual-port RAM interface will be located in an internal area of RAM of the iMX6 SX device.  It’s format and implementation will be exactly the same as the EN100(+) interface with the exception of how the inter-processor interrupts are generated and cleared.  Instead of read/writing to the interrupt cells of the dual-port RAM (last 2 locations of memory) the cores will communicate via their internal inter-processor communication system, i.e. interrupts.

### M4 Flash - Memory Map

Figure 9 – M4 Flash - Memory Map

### Third party components

Following table provides information on third party components used on the M4 core.

Table 3-3: Third party components

| Component | Description |
| --- | --- |
| File manager | Segger System emFile Flash Memory filing system is incorporated |
| Communication Manager | Triangle Microworks DNP 3.0 Protocol Stack is incorporated |
| Operating System | Whittenstein OpenRTOS version of FreeRTOS |
| Graphics Driver | Segger System emWin graphics library. This will be the free version provided by NXP/Segger. |

## 7SR5 Device Architecture

## Reydisp Manager Template Generation

7SR1 and 7SR2 Reydisp Manager Templates are generated using a combination of user definitions and device interrogation.  Device interrogation involves locating suitable hardware, loading the target product installer, downloading the runtime configuration (settings, setting values, Booleans, etc). Due to the number of hardware variants for a product, this process is time consuming.

7SR5 Reydisp Manager Template generation will remove this time consuming process by making the device state deterministic by adding additional definitions in the Reylogic Toolbox project for a product.

### Template Input Format

### Template Output Format

### RDS/RDO Processing

7SR1 and 7SR2 template generation relies upon a 61850 sections being defined in RDO. This section contains an XML-like representation of the IEC 61850 SCL tree for the RDO.  The SCL trees for each RDO defined in the product RDS are collected and used as input into the ICD generation process.

This process will be modified to serve the purposes of 7SR5 template generation with these overall goals:

- Use standard and established XML formats and technologies.

- To increase the expressive power of the RDO data.

- To allow the RDO to define not only SCL for the template process, but also relay settings, user logic, events, etc.

- To allow ad-hoc and dynamic platform descriptions of SCL, relay settings, user logic, events, etc to be generated by Reydisp Manager 2.

- To include validation mechanisms that the RDO data is defined correctly before use.

As a result, the 61850 section of an RDO will contain more than just 61850 definitions.  This section should be renamed to correctly reflect the intended contents.  However, keeping the 61850 section name requires less modification of existing libraries.

#### RDO XSL

The 61850 section of RDO files will be modified to contain only valid XML.  A subset of  XSL 1.0 can be used within the XML.  The XML tree will be extracted, automatically converted to an XSL stylesheet, and evaluated as an XSL transformation, the output of which will be validated against an XML Schema (XSD).

RDL OBJECT DEFINITION OBJECT NAME FOO OBJECT DESCRIPTION Just an example RDO. OVERVIEW A demo RDO. 61850 <RDO xmlns="http://tempuri.org/RdsDataSchema.xsd" xmlns:xsl="http://www.w3.org/1999/XSL/Transform"> <Settings> <Menu Id="{$Name}" Menu="FOO PROTECTION" Description="{$Name}" /> <SimpleSetting Id="FOO.{$OBJNAME}.Setting" Description="{$Name} Setting" Menu="{$Name}" Default="{$DefaultLevel}" Units="{$Units}" Common="false"> <DisplayRule> <MinStepNext><xsl:value-of select="$SettingRange" /></MinStepNext> </DisplayRule> </SimpleSetting> </Settings> </RDO> PROPERTIES STRING "Name" DEFAULT "Foo Name" HELP "The name." STRING "Units" DEFAULT "s" HELP "Units for foo setting." STRING "Setting Range" DEFAULT "0,1,20" HELP "Range for foo setting." FLOAT "Default Level" DEFAULT "2" HELP "Default value for foo setting." INPUTS OUTPUTS RULES END.

Example RDO including XML 61850 section. The example shows menu and setting definitions, the use of RDO property variables using XSL attribute value templates, the use of XSL variables, and the use of xsl:value-of.

FOO MY_FOO, Name = “My Foo”, Units = “ms”, Setting Range = “5,5,1000”, Default Level = 500

Example definition of above RDO in an RDS file. Each property has been changed from its default value.

For each RDO declared in an RDS, the following process happens:

- Locate the correct RDO class for the declaration.

- Instantiate the RDO class

  - Create a representation of an RDO object based on the class.

  - Set all RDS properties for the RDO object.

  - Apply all RDO rules from the RULES section of the RDO.  This may change the value of some properties.

- Extract RDO XML

  - Fetch the XML from the RDO.

  - Convert the XML into an XSL stylesheet (see example below).

  - Execute the XSL transformation.

- Validate the XML output with an XML schema.

<stylesheet version="1.0" xmlns:RDO="Reyrolle:RDO" xmlns="http://www.w3.org/1999/XSL/Transform"> <variable name="OBJNAME" select="RDO:Name()" /> <variable name="OBJTYPE" select="RDO:Type() " /> <variable name="Name" select="RDO:Property('Name')" /> <variable name="Units" select="RDO:Property('Units')" /> <variable name="SettingRange" select="RDO:Property('Setting Range')" /> <variable name="DefaultLevel" select="RDO:Property('Default Level')" /> <template match="/"> <RDO xmlns="http://tempuri.org/RdsDataSchema.xsd" xmlns:xsl="http://www.w3.org/1999/XSL/Transform"> <Settings> <Menu Id="{$Name}" Menu="FOO PROTECTION" Description="{$Name}" /> <SimpleSetting Id="FOO.{$OBJNAME}.Setting" Description="{$Name} Setting" Menu="{$Name}" Default="{$DefaultLevel}" Units="{$Units}" Common="false"> <DisplayRule> <MinStepNext><xsl:value-of select="$SettingRange" /></MinStepNext> </DisplayRule> </SimpleSetting> </Settings> </RDO> </template> </stylesheet>

Example intermediate output when converting RDS XML to an XSL stylesheet. Things to note: - xmlns:RDO="Reyrolle:RDO" An extension object is made available via the namespace prefix RDO. This gives access to the RDO object being evaluated, such as: - RDO:Property(‘Property Name’) evaluates to the set value of the property called ‘Property Name’. - RDO:Input(‘Input Name’) evaluates to the name of the RDO object connected on the input called ‘Input Name’. A special input name ‘$’ can be used to refer to the main input which shares the name of the RDO object. - An XSL variable will be generated for each RDO property to allow property value lookups, e.g. RDO:Property(‘Default Level’), to be shortened to, e.g. $DefaultLevel. A conversion process to turn RDO property names into valid XSL variable names: - Any spaces are removed. - Replace any character that is not alphanumeric, hyphen (-), period (.), or underscore (_), with an underscore (_). - If the name doesn’t start with a letter of an underscore (_), prefix the name with an underscore (_). - Two special variables are created: - $OBJNAME is the name of the object in the RDS. - $OBJTYPE is the class name of the object. - The RDO element of the RDO XML is inserted verbatim into an xsl:template. Thus, any XSL operation that is valid inside al xsl:template, is also valid within the RDO xml, such as: - xsl:if, xsl:choose, xsl:when, xsl:value-of, xsl:variable, etc This stylesheet can then be evaluated to produce the desired output tree for the RDO object being evaluated.

When the above RDO and RDS declaration are evaluated, the following output is expected:

<RDO xmlns="http://tempuri.org/RdsDataSchema.xsd" xmlns:xsl="http://www.w3.org/1999/XSL/Transform"> <Settings> <Menu Id="My Foo" Menu="FOO PROTECTION" Description="My Foo" /> <SimpleSetting Id="FOO.MY_FOO.Setting" Description="My Foo Setting" Menu="My Foo" Default="500" Units="ms" Common="false"> <DisplayRule> <MinStepNext>5,5,1000</MinStepNext> </DisplayRule> </SimpleSetting> </Settings> </RDO>

Example of RDO evaluation when applied to the RDS declaration: FOO MY_FOO, Name = “My Foo”, Units = “ms”, Setting Range = “5,5,1000”, Default Level = 500

#### RDS Processor Output

A new RDS Processor will be created to extract RDS/RDO data (RdsData).  The existing ICDGenerator will be modified to support this data as input to the IXD/ICD generation process.  The RdsData will also be used by a new 7SR5 Template Generator tool to generate 7SR5 Templates.

XML Schema for RdsData (TODO: Insert link here).

##### Development tool RDS/RDO Validation

### Dynamic SCL

TODO

# General Architectural Topics

## Release Classification & Version Planning

The release classification and numbering will adhere to the guidelines laid out in the Siemens official Release Classification document here:

I:\General-Data-D\Bln\Q_Docu\PLM_Lean\Process\04_Guide-CM_Release_Classification\
PLM_TU_ReleaseClassification_EN.pdf

The FW and Devices will have the same version number to simplify things for developers and customers.

- Prototypes up until type test/product release will be V1.xx.xx

- First type test candidate will be V2.00.00

- First released product will be V2.00.03 after defect corrections

In this way it is not seen as a first version product which it should not be as it will have been tested many-many times.

The RDM version number will remain separate at V2.xx.xx upon final release.

With the FW/Devices/RDM all at V2.00 it would give us the potential to keep everything in step – V2.x RDM and V2.x 7SR5 are fully compatible. And increment them where required.

## File Formats – CMS/VOL/KON/PCK

This section gives a brief description of each file format and what they are used for.

### File Formats – CMS

- The CMS is the output of the digital signing process.

- This will be the result of signing a firmware or a device configuration.

- This is an intermediate file and will not be uploaded to the device.

### File Formats – VOL

- User configurations will use a VOL output file format.

- VOL files are not digitally signed, as this would require the customer to have access to digitally sign software and configuration packages.

- VOL files can only be uploaded to the device via the HTTPS loading mechanism.

### File Formats – KON

- The KON file will be the output file format for firmware and device configurations.

- Once the file has been digitally signed as a CMS, then it will be converted to a KON which can be uploaded to the device.

- KON files can be loaded via the DTLS loader or the HTTPS loading mechanism.

### File Formats – PCK

- The PCK is used to package multiple KON files into a single deliverable file.

- PCK files can be uploaded via the DTLS loader or the HTTPS loading mechanism.

## Deliverable Packages

This section describes the various packages that will be produced during the 7SR5 platform lifecycle.

### Deliverable Packages – Protection Firmware

- The protection firmware refers to the M4 firmware and the configuration scripts.

- Generally, it will only include the M4 firmware and configuration scripts, but where logical it can include other files such as HWT and the BootRom.

- Configuration scripts are system configurations that are uploaded into the SYSTEM area of the file system on the device.

- These are RDL scripts and configuration files that define protection elements and functionality within the device.

- The protection firmware package contains digitally signed files and can be loaded via DTLS or HTTPS.

$/PRODUCTS/518_RN/7SR5_Platform_2518H80022/

### Deliverable Packages – Communications Firmware

- The communications firmware refers to the A9 firmware.

- This firmware is currently produced by Nanjing.

- It provides a Virtual EN100 interface for the device, and the software is based on the EN100 code.

- Generally, the Communications package will include the A9 firmware, but it can also include the BootRom and FPGA images, should they need to be upgraded.

- In the future, it is planned that this will include regular security updates.

- The communications firmware package contains digitally signed files and can be loaded via DTLS or HTTPS.

$/PRODUCTS/518_RN/7SR5_CommsFirmware_2518H80024/

### Deliverable Packages – User Configurations

- User configurations are produced using Reydisp Manager.

- They allow the user to commission a device and setup the protection elements and functionality in a device.

- A user configuration package is not digitally signed, so it can only be uploaded using HTTPS (as it provides a secure communications protocol).

### Deliverable Packages – Production

- There are three HEX files that are required for production.

- The hex images are flash images for the two external flash parts on the main CPU board.

| Name | Description |
| --- | --- |
| 7SR5_M4HwtPcba_FlashImgM4.hex | M4 flash part image |
| 7SR5_M4HwtPcba_FlashImgA9_Electrical.hex | A9 Electrical flash part image |
| 7SR5_M4HwtPcba_FlashImgA9_Optical.hex | A9 Optical flash part image |
| A9FW | 0_Sign_A9FW.bat |
| M4FW | 0_Sign_M4FW.bat |
| M4HWT | 0_Sign_M4HWT.bat |
| Device Scripts | 0_Sign_M4SYS.bat |

Table 4-1 Manufacturing hex file deliverables

#### Building intermediate PCK images

- The hardware test PCK files are used to create the hex images deliverables.

- Simply run “1_Build_7SR5_M4HwtPcba.bat” to create the “Electrical.pck” and “Optical.pck” files.

- All the files required can be found in Vault:

$/PRODUCTS/518_RN/7SR5_M4HwtPcba_2518H80023/1_build_pck/

#### Building production deliverable HEX images

- The Prerequisite hardware to build the HEX images is as follows: - processor cards (electrical and optical), PSU and fascia.

- Using additional cards will create additional files to be stored in the HEX images, such as calibration files from slave cards.

- The files required to build the hex image can all be found in Vault:

$/PRODUCTS/518_RN/7SR5_M4HwtPcba_2518H80023/2_build_hex/

- Prerequisite: Update CMD files to ensure the correct Segger file path is used, based on the version of software installed on the local PC – for example: JLink_V614h

- Attach debug connector and the front USB cable

- Place the device into recovery mode by pressing the three buttons simultaneously on startup – Down, 1 and 0.

- Wipe the current A9 image by running “Run WipeA9Flash.cmd” [~2mins]

- Wipe the current M4 image by running “Run WipeM4Flash.cmd” [~2mins]

- Load the new BootRom by running “uploadbootrom.cmd” [~20secs]

- Power cycle the device and wait for the flash to be formatted

  - While formatting it will draw a slightly higher current ~190/260mA [~40secs]

  - Once the format is complete the current will drop ~180/250mA

- Power cycle the device again and restart in recovery mode, as described above.

- Load the relevant PCK (“Electrical.pck” or “Optical.pck”)

- Restart the device and ensure that the file system is formatted during startup - DOC format message will appear.

- Power cycle the device and start recovery mode again.

- Open the J-Flash tool and load the project “A9FS.jflash”

- Connect to the device in J-Flash.

- Read the A9 image using the manual read option with the address range 0x50000000 – 0x50FFFFFF. Then export the memory dump as a HEX and BIN file [~1min]

- Read M4 image using the manual read option with the address range 0x52000000 – 0x52FFFFFF. Then export the memory dump as a HEX and BIN file [~1min]

- Repeat the process for both optical and electrical images

## Device Upload – Firmware and configurations

### DTLS loader – Recovery Mechanism (FMU)

- This method uses the BootRom to load packages into the device, using the SIPROTEC Firmware Upload program (FMU).

- The DTLS loader/FMU mechanism will only be used for recovery purposes, when the relay is returned to an authorized SIEMENS repair center – this may be the original manufacturing factory or a regional office – FMU with DTLS support is an internal tool and we do not wish to distribute it to customers.

#### Installing packages with FMU

- The device must be in boot mode to upload packages via the FMU.

- To enter boot mode: press and hold the DOWN, 1 and 0 keys simultaneously and power the device on.

- The device will now be held in boot mode and will not attempt to startup.

- Only digitally signed firmware and configurations scripts can be loaded using FMU.

- PCK containing signed KON files and individual signed KON files can be uploaded with FMU.

- VOL files and unsigned KON files cannot be uploaded using FMU.

### HTTPS loader – Device Healthy upload mechanism

- While the device is running the HTTPS loader is active and packages can be uploaded to the device.

- The HTTPS loader will upload digitally signed firmware, configurations scripts and user configurations.

- PCK packages containing signed KON files can be uploaded via HTTPS.

- User configurations (user.vol) can be uploaded via HTTPS.

- Only PCK and user VOL files are recognized by the HTTPS loader – KON files are not accepted.

- It is possible for the HTTPS to accept unsigned packages (PCK packages that contain unsigned KON files). However, this will only be used in development and will be disabled on all customer devices.

## Firmware Signing & Verification

- The firmware signing is a security feature so that only signed firmware or configuration scripts will be accepted by the device.

- Firmware signing is required for all binary firmware packages: - 
	BootRom, 
	FPGA, 
	A9FW, 
	M4FW and 
	M4HWT.

- In addition, system configuration scripts must also be signed.

- User configurations are not signed.

### Firmware Signing – Overview

- A PC with a special setup is used to access the signing server in Germany.

- The current PC is named MD1RG6MC, and all the tools are set up.

- Four people have access to the signing server: - 
	Marcus Bainbridge, 
	Neil Dosdale, 
	Keith Stenson and 
	Malcom Bloodworth.

- This is the same signing process as Nanjing and was setup with their help.

### Firmware Signing – File Formats

- The signing process uses a binary file as input and produces a signed binary file. The signed binary can then be used as normal.

- Firmware will be in the BIN format and output a CMS signed binary format file. This file is then converted to a KON file, which can then be uploaded to the device.

- Device configuration scripts use a raw uncompressed VOL format and output a CMS signed volume format file. This file is then converted to a KON file, which can then be sent to the device.

### Firmware Signing – PC Setup

- The install files can be found in 5_DEVELOPMENT Vault, here:

$/PLATFORMS/518/FIRMWARE/FirmwareSigning/

### Firmware Signing – Signing Process

- The process for signing firmware or scripts remains the same for all items.

- Remote desktop/Log In to laptop - MD1RG6MC (2019-07-04: All tools are setup to run on PC)

- Get the files to sign from Vault.

- Run the appropriate signing .BAT file.

- Store results back in Vault.

- Log off remote PC.

| Name | Vault Location | File to run |
| --- | --- | --- |
| BootRom | $/PLATFORMS/518/FIRMWARE/BootRom_2518S80611/SIGN/ | Not signed in Hebburn |
| FPGA-E | $/PLATFORMS/518/FIRMWARE/FPGA_Electrical_2518S80612/SIGN/ | Not signed in Hebburn |
| FPGA-O | $/PLATFORMS/518/FIRMWARE/FPGA_Optical_2518S80613/SIGN/ | Not signed in Hebburn |
| A9FW | $/PLATFORMS/518/FIRMWARE/A9FW_2518S80610/SIGN/ | 0_Sign_A9FW.bat |
| M4FW | $/PLATFORMS/518/FIRMWARE/Mod518_V1_2518S80031/SIGN/ | 0_Sign_M4FW.bat |
| M4HWT | $/PLATFORMS/518/FIRMWARE/HWT_2518S80471/SIGN/ | 0_Sign_M4HWT.bat |
| Device Scripts | $/PRODUCTS/518_RN/RN 2518H80xxx/SIGN/ | 0_Sign_M4SYS.bat |

Table 4-2 Firmware Signing locations

### Firmware Signing – Disable Firmware Signing (For Development Only)

- The HTTPS firmware signing checks can be disabled on startup, and shall only be used on development devices within R&D.

- The DTLS (recovery mode) firmware signing check cannot be disabled.

#### Firmware Signing – Enable unsigned mode

- To enable the unsigned firmware mode, the following procedure is required:

- The device must be started in hard debug mode. (Hard debug mode uses the file stored in secure, while the soft debug mode is access via the boot menu)

- Enter the password command over the ASCII comms interface “PW”

- Enter the debug command “SIGNSIGNSIGN”. This will return “Done” if successful.

- An INF entry will be added to the FACTORY area on the file system.

- Restart the device

- Unsigned firmware will not be accepted – the device must remain in debug mode (hard or soft) for the to be disabled.

#### Firmware Signing – Disable unsigned mode

- To remove the unsigned mode and enable the firmware signing checks, follow the same procedure and call the “UNSIGN” debug command, instead of the “SIGNSIGNSIGN” command.

## MAC Addresses

- The MAC address is usually programmed into the main processor card during the manufacturing process.

### MAC Addresses – Creating a MAC in development

- A MAC image can also be created and programmed in development.

- The files to create a MAC address can be found in Vault:

$/PLATFORMS/518/CONFIG/MacCreate_2975H80049/Release/

- The MAC address image will need to be digitally signed, so the signing server needs to be used.

- The MAC address image can be loaded by HTTPS or the DTLS loaders.

- The following process can be used to program a MAC address during development:

- Log on to the signing PC - MD1RG6MC (you will need to have access rights to the signing server).

- Get latest files from Vault folder.

- Edit 0_CreateAndSignMacBinary.bat to include the appropriate MAC, serial number and article number.

- Run 0_CreateAndSignMacBinary.bat

- Store the output PCK – by default this is mac_cms.pck.

### MAC Addresses – HBB R&D Assigning a MAC address

- HBB R&D have been allocated one hundred MAC addresses from GOA MF database.

- These are predefined MAC addresses and the package to install each one has already been created and digitally signed.

- They are stored in Vault.

$/PLATFORMS/518/CONFIG/MacCreate_2975H80049/Stored/OfficialMacs/

#### Allocating an unused MAC address

$/PLATFORMS/518/CONFIG/MacCreate_2975H80049/Stored/OfficialMacs/AllocateMacAddress.xls

- Update the spreadsheet to allocate the next available MAC address to the device.

- Then rename the allocated MAC in Vault to include the serial number of the device.

- Load the MAC using either the HTTPS loader or DTLS loader.

- When the device restarts is should display “New MAC detected” on the first startup screen.

## Watchdog Mechanism

Question as below:

- Hardware watch dog should be kicked by whom? A9 or M4?

- Can A9 monitor M4 and reset it?

## Exception handing and error processing

Question as below:

- What is behavior while some error occurs? Such as parameters is null?

## Endian Format

- EN100’s Endian format is big-endian, but the IMX6SX only support little-endian.

# A9 Firmware Architectural Topics

## A9FW – CMP

To follow…

## A9FW – Board Programming

The instructions below describe the process for programming the A9 Core code on the main CPU board on the 7SR5 platform.

### Prerequisite Tools

#### USB Driver (DIGSI USB Driver)

The DIGSI USB drivers are used to program the device in boot mode via the USB port. If you do not have the DIGSI USB drivers already installed, then they can be found here (Siemens DIGSI V4.88 Drivers):

I:\Proj-Data-D\Hbb\351482_Reyrolle_7SRn_Native_61850\P.M100-P.M380_GeneralSubjects\Planning
\NKG\Swap\20170825_Loaders\UsbDrivers\Setup.msi

### KON file loader

The KON file can contain a whole flash image or an image for a single flash partition.

With respect to the firmware this is an executable binary that has been converted into a flash image.

The firmware upload program can be copied onto your local driver and run from there.

You can also associate it with KON files so that double clicking on a KON file will automatically open the firmware update tool, which can be found here:

I:\Proj-Data-D\Hbb\351482_Reyrolle_7SRn_Native_61850\P.M100-P.M380_GeneralSubjects\Planning
\NKG\Swap\20170825_Loaders\FIRMWAREUPDATE.EXE

#### Uploading a KON file

- Power off the relay.

- Connect a USB cable.

- Start the relay up with the DOWN key pressed – and hold this for 10 seconds to put the relay into boot mode.

- Load the desired KON file into the firmware update tool.

#### Configuring your COM port

Note from JS: 
My Siprotec driver installed as COM25. The only way I could set the upload tool to use this by

default was to modify the FirmwareUpdate.ini file (mine was in C:\Users\john.sprague\AppData\Roaming\SIEMENS\Siprotec\Firmware) setting ComPort=25. Then with the device in upload mode I could double click the KON file and load it.

### Boot Fuses

A brand new board will not have the boot fuses programmed (burned in), so the processor card will not be able to boot from the flash.

At the moment the way to burn the fuses is by programming the A9 core with the Hebburn A9ToStartM4 code, using the IAR debugger and J-Link. This IAR project can be found in Vault:

$/5_DEVELOPMENT/PLATFORMS/518/FIRMWARE/A9ToStartM4/A9ToStartM4.eww

### Boot ROM

#### First Time Programming

On a blank board the A9 Boot ROM will need to be programmed for the first time using the IAR Debugger and J-Link tools. The initial Boot ROM can be found here:

I:\Proj-Data-D\Hbb\351482_Reyrolle_7SRn_Native_61850\P.M100-P.M380_GeneralSubjects\Planning
\NKG\Swap\NKG Binary\NKG_Binary.eww

After the Boot ROM has been initially loaded then it is possible to program it the normal KON file method described above in Section 5.2.2.

#### Normal Programming

After the Boot ROM has been initially loaded then it is possible to program it the normal KON file method described above in Section 5.2.2.

The latest Boot ROM can be found here:

I:\Proj-Data-D\Hbb\351482_Reyrolle_7SRn_Native_61850\Project-NoArchive\EN100\Firmware
\bootrom.kon - latest.lnk

### FPGA

The FPGA is programmed using a KON file, which is described above in Section 5.2.2.

The latest FPGA code can be found here:

I:\Proj-Data-D\Hbb\351482_Reyrolle_7SRn_Native_61850\Project-NoArchive\EN100\Firmware
\fpga.kon - latest.lnk

### Network Settings

At the moment we can only connect directly from the PC to the Ethernet port.

- To do this you will need to configure the TCP/IP v4 settings for this NIC (ip address 192.168.0.11; netmask 255.255.255.0, no gateway).

- Connect the USB adapter directly to the top RJ45 socket on the relay.

- You should see the green light on the CPU board come on.

- The default IP address for the relay is 192.168.0.4

### VxWorks

The VxWorks image is programmed using a KON file, which is described above in Section 5.2.2. The latest image can be found here:

I:\Proj-Data-D\Hbb\351482_Reyrolle_7SRn_Native_61850\Project-NoArchive\EN100\Firmware
\vxworks.kon - latest.lnk

### M4 Binary

The M4 firmware will also produce a KON file that can be downloaded.

# M4 Firmware Architectural Topics

## M4FW – CMP

The full CMP document for the 7SR5 platform can be found here:

7SR5 Configuration Management Plan - CMP_7SR5_Platform_SW.doc
$/518_7SRn_Native_61850/Software/7SR5 Platform/CMP_7SR5_Platform_SW.doc
V:\6\518_7SRn_Native_61850\Software\7SR5 Platform\CMP_7SR5_Platform_SW.doc

### Software Project Shell (SPS)

Software Project Shell V3.41.00.10 has been released to support the new 7SR5 platform tools. There are three new tool files relevant to the 7SR5 platform:

- iMX6SX_M4_Application.tl		- Main processor application

- iMX6SX_M4_Library.tl			- Main processor library

- K22F_SlaveCard_Application.tl		- Slave card application

SPS Version Number:
2974H80001 R017 - Software Project Shell 3.41.00.10

Vault Location:
$/974_SoftwareProjectShell/2974H80001 R017 - Software Project Shell 3.41.00.10/

### IAR Compiler

All firmware on the 7SR5 platform is built using the IAR compiler V7.70.1.

Once the compiler is installed the network license needs to be manually configured using the information below.

Figure 10 – Manual configuration of the IAR network license.

Compiler Version:
IAR ANSI C/C++ Compiler V7.70.1/W32 for ARM

Install Location:
Y:\437_Tools\v7.70.1\EWARM-CD-7701-11486\ew\setup.exe

Network license:
iar-lic-debwg01.siemens.net

### J-Link Debugging Tools

The SEGGER J-Link debug tool is used to debug on the 7SR5 platform and will require V6.14e of the J-Link software, which can be found here:

Y:\437_Tools\J-Link\JLink_Windows_V634c.exe

## M4FW – Slave Card Programming

This section describes the process to program the slave cards personality.

### Basic Card Programming

- The personality of a slave card is programmed using an ASCII terminal program.

- The default COM port options are: 57600, No Parity and 8 Stop bits.

- The programming sequence and commands are listed below:

Login in:		CW 866315

Enter MF mode:	start_mf_prog	                    // Forces a restart

Login in:		CW 866315

Set Personality:	CPUID  <Article Number>  <Unique Serial Number>

Exit MF mode:		quit_mf_prog

### Analogue Card Programming

In addition to the basic personality programming (as described above) the analogue cards need to have their calibration setup. The basic process to zero the calibration should be performed on all new cards, if they are not going to be calibrated immediately. The process is listed below:

Login in:		CW 866315

Reset calibration:	ZEROCAL

Store calibration:	STORECAL

### Slave Card Article Numbers

| Name | Description | Article Number |
| --- | --- | --- |
| CT Card | 4 CT + 3BI | C53040-A70-C77-x |
| VT Card | 4 VT + 1BI + 2BO | C53040-A70-C78-x |
| IO Card | 5BI + 2BO | C53040-A70-C79 x |
| PSU Card | PSU + 4BO | C53040-A70-C75-x |
| S6 Fascia Card | S6,128x128LCD,7K,28L | C53040-A70-C73 3 |
| S12 Fascia Card | S12,128x128LCD,7K,28L | C53040-A70-C72 3 |
| S8 Fascia Card | S8,128x128LCD,7K,28L | C53040-A70-C88 1 |
| ARC Card | ARC PROTECTION WITH HS BO | C53040-A70-C84 2 |
| TSI Card | TSI | C53040-A70-C80 2 |
| HDBO | MK22 5BI 2HDBO | C53040-A70-C93 1 |
| S6 (LPC55) | S6,128x128LCD,7K,28L LPC55 | C53040-A70-C97 1 |
| S8 (LPC55) | S8,128x128LCD,7K,28L LPC55 | C53040-A70-C96 1 |
| S12 (LPC55) | S12,128x128LCD,7K,28L LPC55 | C53040-A70-C95 1 |
| IO (LPC55) | 5BI + 2BO LPC55 | C53040-A70-C94 1 |
| PSU (LPC55) | PSU + 4BO LPC55 | C53040-A70-C98 1 |

Table 6-1 Slave Card Article Numbers

## M4FW – Sampling

### Analogue Input RDL naming convention

The raw inputs for the CT and VT cards will be named as follows:

| Name | Description |
| --- | --- |
| AN_I1 … AN_In | Phase Current Input |
| AN_S1 ... AN_Sn | Earth / SEF Input |
| AN_V1 … AN_Vn | Voltage Input |

Table 6-2 Analogue Input RDL naming convention

This will be compatible with the existing RM and RC platforms.

## M4FW – Timers

## M4FW – Hardware timers

## M4FW – Real Time Clock

# 7SR5 Devices Architectural Topics

# Reydisp Manager Architectural Topics

## Reydisp Manager Template Dataflow

## Dynamic Setting View/Edit

Reydisp Manager 1 uses a settings file in RSF format to store settings.  The source of the RSF file is a running device that is configured with the target MLFB.  To modify a relay setting value, the target setting is found on the device by the index which specifies its location in the settings file; this means that the settings file must always be kept in step with the relay.  A validity check of the settings file and relay setting database is performed by a CRC which checks that the structure matches.  If the CRC validation fails, the entire setting database must be retrieved again from the relay.  Any pending user modifications must then be re-applied to the settings and then the process re-attempted.

Producing device templates for Reydisp Manager 1 requires that settings files must be retrieved from running devices for a product release.  This enables the user to configure the device offline without having been connected to it.

Handling settings in RDM2 for 7SR5 devices shall be modified with these goals:

- A setting tree should be deterministic from template data without the need to connect to a running device.

- Setting values should be communicated with a unique identifier, rather than by setting index (the position of the setting in the setting database).

- Setting values should be stored with only minimal data that can be applied to multiple IED configurations.

  - Setting ID

  - Setting Value (raw; value index, for example)

  - Setting Value (display; setting value displayed to user)

  - Setting Group (the group the value is applied to)

- The setting tree should dynamically represent the current IED configuration.  Considerations to be made are:

  - The IO for the IED will be determined by the MLFB.  Any settings which directly reference IO (status inputs, binary outputs, LEDs, function keys, etc) will be generated to reflect the current IO configuration:

    - Output/Input matrix bitfield settings should have the correct size and bit names.

    - Any IO configuration settings (output delays, etc) should be generated multiple times; once for each IO instance (BO1, BO2, BO3, etc).

  - Any setting values which rely upon device configuration state will be generated to reflect that state.

    - User curves which are settable as a characteristic setting (for an IDMTL for example) should be selectable values.

    - Language – TODO: Don’t know how language selection is happening.

  - Any setting that relies upon an element list (trip storage, for example) shall be generated based on which functionality is currently selected by user configuration.

    - TODO: How to know which elements go into which trip storage setting, for example?

Figure 11 - 7SR5/RDM Settings Sequence

## Complex Settings

Some settings value in definition depending on the MLFB (IO, etc) or configurations state.  Additional considerations need to be made when creating a setting of these types.

### I/O Matrix Bitfield settings

Settings that define the IO (binary inputs, binary outputs, LEDs, virtuals, Fn Keys, etc) will change in definition depending on the MLFB that is used to initialize them, each bit in the bitfield representing IO that is present. The IO configuration for the selected MLFB will be given to the settings tree generator so that it may correctly initialize I/O matrix bitfield settings.

### Or Masks (Pickup Config) Settings

Pickup config settings contain a list of bits for each element of a given function group.  The relay RDL builds these settings by RDL BOOL registering a bit with an RDL ORMask object (by name).  This process also needs to be performed in the Reydisp Manager template libraries for generating settings trees.

Implementation notes:

In RDL BOOL: <RegisterOrMask OrMaskName=”{$Str OrMsk Name}” BitName=”{$TagName}” />

Im RDL ORMSK:

<SimpleSetting  …>

<DisplayRule>

<CollectOrMask OrMaskName=”{$OBJNAME}” />

</DisplayRule>

</SimpleSetting>

## Reydisp Manager 7SR5 Template Workflow

| Product Definition |  | Product Template |  | Device Template |
| --- | --- | --- | --- | --- |
| Contains description of product, including: Protection functionality, IO description, MLFB structure, Rules for element CT/VT mapping restrictions. | A publish process, 7SR5-Template generator, processes and validates the input data, collects RDS, collects RDO information, collects SCL fixed indications | Contains all template information required to configure an IED for every MLFB in the product. | Filters the product template down to what is required for a specific target MLFB (defined by Reydisp Manager). Filters the protection list down to what is specified by the user in Reydisp Manager. Applies element CT/VT mappings specified by the user in Reydisp Manager. Generates settings tree, user logic signals, events, SCL (etc) that are specific to the target MLFB and user configuration. | Contains:- Settings tree (with default values), User logic signals, SCL (IXD with default IED name and configuration), Events, RDS/RDO (internal representation to be converted to RDS). |

# References

- 7SR5 Configuration Management Plan - CMP_7SR5_Platform_SW.doc
$/518_7SRn_Native_61850/Software/7SR5 Platform/CMP_7SR5_Platform_SW.doc
V:\6\518_7SRn_Native_61850\Software\7SR5 Platform\CMP_7SR5_Platform_SW.doc